SMART INDIA HACKATHON 2026

Problem SIH26170  ·  Theme: Smart Automation  ·  Organisation: ISRO / Department of Space

AETHER

Adaptive ESS Thermal Health & Early Reject

Sources and references used to come up with the solution

| Item | Detail |
| --- | --- |
| Document type | Bibliography and source map (IEEE numbered) |
| Companion reports | AETHER_SIH26170_Technical_Report.docx; AETHER_ML_Modelling_Desktop_and_Web_Catalogue.docx |
| Date | 12 September 2026 |
| Purpose | Cite every standard, paper, and dataset that shaped Module A, Module B, burn-in cadence, and evaluation |

AETHER was not trained on a public ISRO flight dataset. Real screening data for space-grade parts is not published. The design starts from the SIH26170 problem statement, then maps onto automotive and military screening standards, IDDQ / burn-in literature, and standard machine-learning estimators implemented in scikit-learn. Training lots in data/burnin_parts.csv are physics-informed synthetic data generated by src/generate_data.py.

# 1.  How the sources were used

Each citation below is tied to a concrete design choice in the code or the evaluation protocol. Nothing is listed only because it is a well-known paper in the field.

| AETHER piece | Source used |
| --- | --- |
| Problem to solve: dynamic 45 µA vs 10 µA lot; 0 h + 24 h → 168 h; FN cost; explainability | SIH26170 problem statement [1] |
| PAT k = 6, lot-relative robust limits, mavericks inside the datasheet box | AEC-Q001 Rev-D [2] |
| 125 °C powered burn-in; 0 / 24 / 96 / 168 h readout cadence | MIL-STD-883 TM 1015 [3]; MIL-PRF-38535 [4] |
| Observables: IDDQ, leakage, propagation delay | Hawkins / Soden IDDQ reviews [5], [6] |
| Isolation Forest on lot-normalised early features | Liu, Ting and Zhou, ICDM 2008 [8] |
| Robust Mahalanobis via MinCovDet | Rousseeuw and van Driessen, 1999 [9]; Mahalanobis, 1936 [10] |
| median + 6 × 1.4826 × MAD | Hampel, 1974 [11]; AEC-Q001 robust sigma [2] |
| Inspectable 168 h forecast (Ridge) | Hoerl and Kennard, 1970 [12] |
| Optional histogram gradient-boosting blend | Friedman, 2001 [13] |
| FN_COST = 1000 vs FP_COST = 12; recall-first thresholds | SIH26170 [1]; Elkan, 2001 [15] |
| Twelve confirmed-defect ICs (2274 … 2713) as a real-world check | SEMATECH / Nigh et al., ITC 1998 [7] |
| Training lots (4,945 parts, 24 lots) | Synthetic generator — not a downloaded dataset |

The twelve SEMATECH devices are scored one-at-a-time against a known-good reference lot in scripts/predict_realtime_iddq.py. They are not one manufacturing lot, so in-lot PAT among themselves would be the wrong test. They are not used as training data.

# 2.  Problem statement

This is the primary specification. Module A, Module B, the 10 / 45 / 50 µA worked example (pinned as LOTSIH-0045), and the three judging metrics all come from here.

[1]  Smart India Hackathon 2026, problem SIH26170, “AI-Driven Anomaly Detection in Component Burn-In & Screening,” organisation: Indian Space Research Organisation (ISRO) / Department of Space, theme: Smart Automation.

https://sih2026.vuce.in/ps/SIH26170

# 3.  Screening standards

Automotive Part Average Testing already names the idea the problem statement asks for: a part that is still inside the datasheet can be a maverick relative to its lot. Military burn-in methods set the temperature and duration that the 0 / 24 / 96 / 168 h cadence follows.

[2]  Automotive Electronics Council, AEC-Q001 Rev-D, “Guidelines for Part Average Testing.” Static PAT = Robust Mean ± 6 Robust Sigma; Dynamic PAT uses the current lot. Used in AETHER as PAT_K = 6.0 and MAD_TO_SIGMA = 1.4826 in src/config.py.

http://www.aecouncil.com/Documents/AEC_Q001_Rev_D.pdf

[3]  U.S. Department of Defense, MIL-STD-883, Test Method 1015, “Burn-in Test.” Purpose: screen infant-mortality failures at elevated temperature (125 °C minimum for conditions A–E). AETHER treats 125 °C as the soak, not as a datasheet check.

https://landandmaritimeapps.dla.mil/Downloads/MilSpec/Docs/MIL-STD-883/std883.pdf

[4]  MIL-PRF-38535, General Specification for Integrated Circuits (Microcircuits) Manufacturing. Class Q typically 160 h and Class V typically 240 h at 125 °C under TM 1015. The problem statement’s 0 h, 24 h, 96 h and 168 h readouts are the practical measurement cadence of that flow.

https://landandmaritimeapps.dla.mil/Downloads/MilSpec/Docs/MIL-PRF-38535/prf38535.pdf

# 4.  Electronics and IDDQ literature

These papers justify why standby current, leakage and delay are the right observables, and they supply the only published real-device table used in this repository.

[5]  J. M. Soden, C. F. Hawkins, R. K. Gulati and W. Mao, “IDDQ Testing: A Review,” in IDDQ Testing of VLSI Circuits. Dordrecht: Springer, 1992, pp. 5–17. IDDQ as a defect-sensitive DC test for bridges, gate-oxide shorts and leakage that stuck-at tests miss.

https://link.springer.com/chapter/10.1007/978-1-4615-3146-3_1

[6]  C. F. Hawkins, J. M. Soden, R. R. Fritzemeier and L. K. Horning, “Quiescent power supply current measurement for CMOS IC defect detection,” IEEE Transactions on Industrial Electronics, vol. 36, no. 2, pp. 211–218, May 1989.

[7]  P. Nigh, D. Vallett, A. Patel, J. Wright, F. Motika, D. Forlenza, R. Kurtulik and W. Chong, “Failure analysis of timing and IDDq-only failures from the SEMATECH test methods experiment,” Proc. International Test Conference, 1998, pp. 43–52. DOI: 10.1109/TEST.1998.743135. Table 1 is the source of the twelve confirmed-defect ICs (2274, 3392, 2795, 2890, 3488, 2663, 1787, 1947, 2968, 3457, 2557, 2713) scored in scripts/predict_realtime_iddq.py. Pre-burn-in IDDQ and post-burn-in values at 6 / 78 / 150 h, with physical defect found. SEMATECH also concluded that IDDQ is not a complete replacement for burn-in — which is why healthy AETHER parts still finish 168 h.

https://doi.org/10.1109/TEST.1998.743135

# 5.  Machine-learning methods

Module A fuses three complementary views: robust PAT, Isolation Forest, and robust Mahalanobis. Module B uses Ridge as the inspectable 168 h model, with an optional histogram gradient-boosting blend only when it reduces validation MAE. Implementations are scikit-learn classes cited in [14].

[8]  F. T. Liu, K. M. Ting and Z.-H. Zhou, “Isolation Forest,” Proc. 8th IEEE International Conference on Data Mining (ICDM), Pisa, Italy, 2008, pp. 413–422. DOI: 10.1109/ICDM.2008.17. Used as sklearn.ensemble.IsolationForest on lot-normalised 0 h / 24 h features (400 trees, contamination 0.06).

https://doi.org/10.1109/ICDM.2008.17

[9]  P. J. Rousseeuw and K. van Driessen, “A Fast Algorithm for the Minimum Covariance Determinant Estimator,” Technometrics, vol. 41, no. 3, pp. 212–223, 1999. Robust covariance of the lot z-vector; Mahalanobis distance flags parts outside the lot ellipsoid (sklearn.covariance.MinCovDet).

https://doi.org/10.1080/00401706.1999.10485670

[10]  P. C. Mahalanobis, “On the generalised distance in statistics,” Proceedings of the National Institute of Sciences of India, vol. 2, no. 1, pp. 49–55, 1936. Distance of a part to the lot centroid under the estimated covariance.

[11]  F. R. Hampel, “The influence curve and its role in robust estimation,” Journal of the American Statistical Association, vol. 69, no. 346, pp. 383–393, 1974. Median and median absolute deviation; the factor 1.4826 makes MAD consistent for the standard deviation of a normal distribution.

https://doi.org/10.1080/01621459.1974.10482962

[12]  A. E. Hoerl and R. W. Kennard, “Ridge Regression: Biased Estimation for Nonorthogonal Problems,” Technometrics, vol. 12, no. 1, pp. 55–67, 1970. Primary Module B model (sklearn.linear_model.RidgeCV) because every coefficient is inspectable in the QA waterfall.

https://doi.org/10.1080/00401706.1970.10488634

[13]  J. H. Friedman, “Greedy function approximation: A gradient boosting machine,” Annals of Statistics, vol. 29, no. 5, pp. 1189–1232, 2001. Optional HistGradientBoostingRegressor blend, used only when it beats Ridge on held-out-lot validation MAE.

https://doi.org/10.1214/aos/1013203451

[14]  F. Pedregosa et al., “Scikit-learn: Machine Learning in Python,” Journal of Machine Learning Research, vol. 12, pp. 2825–2830, 2011. Concrete implementations: IsolationForest, MinCovDet, EmpiricalCovariance, RidgeCV, HistGradientBoostingRegressor, RobustScaler, StandardScaler.

https://jmlr.org/papers/v12/pedregosa11a.html

[15]  C. Elkan, “The Foundations of Cost-Sensitive Learning,” Proc. 17th International Joint Conference on Artificial Intelligence (IJCAI), 2001, pp. 973–978. Decision thresholds follow FN cost 1000 versus FP cost 12 rather than a 0.5 accuracy cut. HOLD exists because space-grade silicon is expensive: a two-class scrap/release policy is the wrong shape.

https://cseweb.ucsd.edu/~elkan/kddcost.pdf

# 6.  Data provenance

## 6.1 Training and held-out lots — synthetic

ISRO / Department of Space screening data for flight parts is not a public download. src/generate_data.py therefore builds lot-structured time series that reproduce the failure modes the problem statement cares about:

- Mavericks — far from the lot centroid, still inside the datasheet box (the 10 / 45 / 50 µA sentence).

- Latent drift — normal at 0 h, anomalous slope, often still in-spec at 168 h.

- Runaway — accelerating degradation that a static limit only catches late.

Healthy parts follow a slow, nearly linear ageing law at 125 °C. Process corners (fast / nominal / slow) shift the whole lot centroid so a global Isolation Forest on raw microamps cannot be used — scores are lot-normalised first. Measurement noise is set to typical parametric-analyser repeatability. Default build: 24 lots, about 4,945 parts, split by lot (not by part) into train / val / test.

## 6.2 Real confirmed-defect ICs — SEMATECH [7]

The twelve-row table pasted into the project (device IDs, pre-BI IDDQ, post-BI IDDQ at 6 / 78 / 150 h, and physical defect) is Table 1 of Nigh et al., International Test Conference 1998 [7]. AETHER maps those currents to microamps and scores each IC against a known-good reference lot (LOT01 healthy parts). Mapping notes live in scripts/predict_realtime_iddq.py.

| Device ID | Physical defect (as published) |
| --- | --- |
| 2274 | Gate-to-drain short (128 kΩ) |
| 3392 | Poly-to-poly short (1.63 kΩ) |
| 2795 | Poly-to-NWell short (340 kΩ) |
| 2890 | Poly-to-NWell short (194 kΩ) |
| 3488 | Metal-to-metal short (184 Ω) |
| 2663 | Poly short in gate-array fill (two independent defects) |
| 1787 | Metal-to-metal short (75 Ω) |
| 1947 | Source-to-drain leakage |
| 2968 | Poly-to-diffusion / substrate leakage across 28 transistors |
| 3457 | Leakage in gate-array fill logic |
| 2557 | Leakage in gate-array fill logic |
| 2713 | Leakage in gate-array fill logic |

These twelve devices are a published check, not the training distribution. SEMATECH also reported that even IDDQ did not fully replace burn-in stressing. AETHER therefore never claims chamber hours back on PASS or HOLD parts; only early REJECT frees 144 h of soak.

# 7.  IEEE reference list (compact)

Copy-ready numbered list for the technical report, poster, or judging pack.

[1]  Smart India Hackathon 2026, SIH26170, “AI-Driven Anomaly Detection in Component Burn-In & Screening,” ISRO / Department of Space. https://sih2026.vuce.in/ps/SIH26170

[2]  Automotive Electronics Council, AEC-Q001 Rev-D, Guidelines for Part Average Testing, 2011. http://www.aecouncil.com/Documents/AEC_Q001_Rev_D.pdf

[3]  MIL-STD-883, Test Method 1015, Burn-in Test.

[4]  MIL-PRF-38535, General Specification for Integrated Circuits (Microcircuits) Manufacturing.

[5]  J. M. Soden, C. F. Hawkins, R. K. Gulati and W. Mao, “IDDQ Testing: A Review,” in IDDQ Testing of VLSI Circuits, Springer, 1992.

[6]  C. F. Hawkins, J. M. Soden, R. R. Fritzemeier and L. K. Horning, “Quiescent power supply current measurement for CMOS IC defect detection,” IEEE Trans. Ind. Electron., vol. 36, no. 2, pp. 211–218, 1989.

[7]  P. Nigh et al., “Failure analysis of timing and IDDq-only failures from the SEMATECH test methods experiment,” Proc. ITC, 1998, pp. 43–52, doi: 10.1109/TEST.1998.743135.

[8]  F. T. Liu, K. M. Ting and Z.-H. Zhou, “Isolation Forest,” Proc. IEEE ICDM, 2008, pp. 413–422, doi: 10.1109/ICDM.2008.17.

[9]  P. J. Rousseeuw and K. van Driessen, “A Fast Algorithm for the Minimum Covariance Determinant Estimator,” Technometrics, vol. 41, no. 3, pp. 212–223, 1999.

[10]  P. C. Mahalanobis, “On the generalised distance in statistics,” Proc. Nat. Inst. Sci. India, vol. 2, no. 1, pp. 49–55, 1936.

[11]  F. R. Hampel, “The influence curve and its role in robust estimation,” J. Amer. Statist. Assoc., vol. 69, no. 346, pp. 383–393, 1974.

[12]  A. E. Hoerl and R. W. Kennard, “Ridge Regression: Biased Estimation for Nonorthogonal Problems,” Technometrics, vol. 12, no. 1, pp. 55–67, 1970, doi: 10.1080/00401706.1970.10488634.

[13]  J. H. Friedman, “Greedy function approximation: A gradient boosting machine,” Ann. Statist., vol. 29, no. 5, pp. 1189–1232, 2001.

[14]  F. Pedregosa et al., “Scikit-learn: Machine Learning in Python,” JMLR, vol. 12, pp. 2825–2830, 2011.

[15]  C. Elkan, “The Foundations of Cost-Sensitive Learning,” Proc. IJCAI, 2001, pp. 973–978.

# 8.  What was not used

- No public ISRO / Department of Space burn-in CSV. Training data is generated, not downloaded.

- The SEMATECH ICs [7] are a held-out real-world check. They are not mixed into train / val / test lots.

- No proprietary ATE STDF from a screening house was ingested. A production line would freeze PAT k and the safety slope on that product after a qualification lot.

End of document. AETHER — Adaptive ESS Thermal Health & Early Reject — SIH26170.